| Source (Asal) | Destination (Tujuan) | Action | Business Reason (Alasan Bisnis) |
| --- | --- | --- | --- |
| Penjualan | SDM | Permit | Requirement: Case B explicitly states these two must coordinate. |
| Penjualan | Gudang | Permit | Assumption: Sales needs to check stock/inventory in real-time before selling. |
| Penjualan | IT / Manajemen | Block | Sales staff should not access server configs or sensitive management files. |
| SDM | Penjualan | Permit | Requirement: Coordination with Sales (recruitment for sales, commissions, etc). |
| SDM | Gudang / IT / Manaj | Block | HR data is confidential; no need to access warehouse or servers. |
| Gudang | Penjualan | Permit | Logic: To reply to stock requests from Sales. |
| Gudang | IT | Permit | Logic: Warehouse systems often need IT maintenance/updates. |
| Gudang | SDM / Manajemen | Block | Warehouse staff should not see payroll (SDM) or executive data. |
| Manajemen | IT | Permit | Logic: To access reports or report technical issues to IT. |
| Manajemen | Penjualan / Gudang / SDM | Block | Executives review reports, they don't log into cash registers or barcode scanners directly. |
| IT (Server) | ALL | Permit | Logic: IT is the "Super Admin". They need to fix problems in every department. |
| Source (Asal) | Destination (Tujuan) | Action | Business Reason (Alasan Bisnis) |

! --- COPY INTO ROUTER UTAMA ---

enable

configure terminal

! 1. ACL for PENJUALAN (Allow to SDM & Gudang only)

access-list 100 remark ACL_PENJUALAN_OUT

access-list 100 permit ip 192.168.10.0 0.0.0.63 192.168.10.64 0.0.0.31

access-list 100 permit ip 192.168.10.0 0.0.0.63 192.168.10.128 0.0.0.15

! (Implicit Deny blocks IT & Manajemen automatically)

! Apply to Interface connected to Switch Penjualan

interface Eth0/3/0

ip access-group 100 in

exit

! 2. ACL for SDM (Allow to Penjualan only)

access-list 101 remark ACL_SDM_OUT

access-list 101 permit ip 192.168.10.64 0.0.0.31 192.168.10.0 0.0.0.63

! (Implicit Deny blocks Gudang, IT, Manajemen automatically)

! Apply to Interface connected to Switch SDM

interface Fa1/1

ip access-group 101 in

exit

end

write memory

! --- COPY INTO ROUTER GUDANG ---

enable

configure terminal

! ACL for GUDANG (Allow to Penjualan & IT only)

access-list 100 remark ACL_GUDANG_OUT

access-list 100 permit ip 192.168.10.128 0.0.0.15 192.168.10.0 0.0.0.63

access-list 100 permit ip 192.168.10.128 0.0.0.15 192.168.10.96 0.0.0.31

! (Implicit Deny blocks SDM & Manajemen)

! Apply to Interface connected to Switch Gudang

interface Fa1/1

ip access-group 100 in

exit

end

write memory

! --- COPY INTO ROUTER MANAJEMEN ---

enable

configure terminal

! ACL for MANAJEMEN (Allow to IT only)

access-list 100 remark ACL_MANAJEMEN_OUT

access-list 100 permit ip 192.168.10.144 0.0.0.31 192.168.10.96 0.0.0.31

! (Implicit Deny blocks Penjualan, SDM, Gudang)

! Apply to Interface connected to Switch Manajemen

interface Fa1/1

ip access-group 100 in

exit

end

write memory

! --- ROUTER SERVER ---

enable

configure terminal

! Allow IT to go everywhere

access-list 100 permit ip any any

interface Fa1/1

ip access-group 100 in

exit

end